| Hazards | Is the hazard present? Y/N | What is the risk? | Risk rating H = High M = Medium L = Low | Control measures | Is this control in place? Y/N | If no, what actions are required to implement the control? | Person responsible | Date action completed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Access to the laboratory |  | Uncontrolled exposure to hazardous properties of laboratory chemicals | H | Access to the laboratory is well controlled, e.g. locked when not in use Unauthorised access is prohibited |  |  |  |  |
| Limited or poor information on hazardous chemicals in use |  | Inadvertent exposure to teachers/ students environment due to unknown hazardous laboratory chemicals | M | Safety Data Sheets (SDS) are readily available for all hazardous chemicals An up-to-date chemical inventory is readily available |  |  |  |  |
| Limited or poor information on labels |  | Inadvertent/ incorrect use of chemicals | M | All hazardous chemicals are labelled correctly in line with the Classification, Labelling and Packaging (CLP) Regulation (Are labels understood, intact and legible?) |  |  |  |  |
|  |  |  |  | Information on the hazard label corresponds to information on the SDS |  |  |  |  |
|  |  |  | H | Containers with non-hazardous chemicals, e.g. water, are clearly labelled to avoid confusion |  |  |  |  |
| Use of hazardous chemicals |  | Exposure to toxic or very hazardous chemicals | H | Elimination/substitution considered, e.g. toxic/ carcinogenic chemicals |  |  |  |  |
| Poor storage arrangements for laboratory chemicals |  | Uncontrolled access and exposure to hazardous laboratory chemicals | H | Chemicals are stored in separate and well ventilated room – access is controlled |  |  |  |  |

| Hazards | Is the hazard present? Y/N | What is the risk? | Risk rating H = High M = Medium L = Low | Control measures | Is this control in place? Y/N | If no, what actions are required to implement the control? | Person responsible | Date action completed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Incorrect storage of chemicals |  | Chemical reaction or exposure to chemicals | H | Chemicals are stored in accordance with the SDS requirements Chemicals are not stored on benches or within fume cabinets |  |  |  |  |
| Chemical spills or reactions of chemicals in storage |  | Chemicals being inadvertently knocked off shelves Chemical(s) with specific storage requirements not being adhered to resulting in an incident |  | Established conventions used, e.g. incompatible chemicals are segregated and stored in compatible hazard classes; Chemicals stored below eye level; Secondary containment is provided for liquid chemicals, anti-roll lips on shelves etc. |  |  |  |  |
| Flammable Chemicals |  | Fire |  | Fire, smoke and heat detectors fitted in store rooms - appropriate fire extinguishers available Quantities are kept to a minimum especially flammable liquids SDSs are readily available in the event of an emergency |  |  |  |  |
| Out of date chemicals in use |  | Chemicals being used outside the manufacturer’s specifications | M | Checks are undertaken at regular intervals, for the purposes of disposal of out-of-date chemicals Chemicals are disposed of in accordance with the SDS and the Local Authority |  |  |  |  |
| Broken glassware |  | Lacerations/ burns/ chemical exposure | H | All glassware and storage vessels are periodically examined for star cracks Correct disposal procedures are in place for glass waste |  |  |  |  |

| Hazards | Is the hazard present? Y/N | What is the risk? | Risk rating H = High M = Medium L = Low | Control measures | Is this control in place? Y/N | If no, what actions are required to implement the control? | Person responsible | Date action completed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Incorrect disposal of chemicals |  | Lacerations/ needle stick injuries and environmental contamination | H | Chemicals are disposed of in accordance with the SDS and the Local Authority. Tick where appropriate Specific storage containers provided for chemical waste Recyclable solvents Designated Sharps Disposal Unit |  |  |  |  |
| Inadequate administrative controls |  | Uncontrolled exposure to hazardous laboratory chemicals to students | H | General laboratory rules are understood and followed by students Particular risks identified for an experiment to be communicated to students Relevant sections of the SDS is conveyed to students Bench tops are clean, organised and environs maintained to eliminate harmful exposures to unsafe conditions See Cleaning (Hazardous Chemicals and Biological Agents) - Template No. 7 |  |  |  |  |
| Lack of knowledge of risks in relation to laboratory experiments |  | Personal injury to persons working in the laboratory | H |  |  |  |  |  |
|  |  | Incorrect use of chemicals leading to chemical exposure | H |  |  |  |  |  |
| Cluttered bench tops and unclean laboratory environment |  | Contact with chemicals | H |  |  |  |  |  |
| Poor personal protective equipment (PPE) controls/ lack of PPE |  | Contamination by chemicals | H | Appropriate PPE is available, e.g. face shields, goggles, lab coats, appropriate hand protection, etc. Students are instructed by teacher before using any equipment |  |  |  |  |

| Hazards | Is the hazard present? Y/N | What is the risk? | Risk rating H = High M = Medium L = Low | Control measures | Is this control in place? Y/N | If no, what actions are required to implement the control? | Person responsible | Date action completed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Inadequate maintenance of fume cupboard |  | Exposure to chemicals/ fumes | H | Fume cupboard fit for purpose and use supervised |  |  |  |  |
|  |  |  |  | Fume cupboard inspected regularly, tested and maintained with test label affixed - records stored appropriately and easily retrievable |  |  |  |  |
|  |  |  |  | Regular checks are made to ensure the average face velocity on these units is adequate (0.5 m/s or higher) |  |  |  |  |
| Poor engineering controls |  | Uncontrolled exposure to hazardous properties of laboratory chemicals | H | SDS consulted for correct engineering controls |  |  |  |  |
| Poor hygiene controls |  | Inadvertent exposure of teachers/ students to hazardous laboratory chemicals | H | General laboratory rules are understood and followed by students, e.g. no eating, drinking, or tasting anything in the laboratory. No food permitted to be brought into the laboratory |  |  |  |  |
| Chemical Ingestion |  | Chemical ingestion resulting in illness or chemical poisoning | H | Safety bulbs must be used when pipetting to avoid inadvertent ingestion |  |  |  |  |
| Ingestion/ absorption of chemicals |  | Illness or chemical poisoning | H | Adequate handwash facilities and eye wash station available, unobstructed and maintained |  |  |  |  |

| Hazards | Is the hazard present? Y/N | What is the risk? | Risk rating H = High M = Medium L = Low | Control measures | Is this control in place? Y/N | If no, what actions are required to implement the control? | Person responsible | Date action completed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Lack of other essential safety items in laboratory |  | Reduced ability to manage emergencies, incidents/ spillages | H | Safety items to be available in the laboratory, e.g. - Safety screen - Fire extinguishers (CO2/Dry powder) - Fire blankets - Fire buckets with sand - Chemical spill clean-up kit -absorbing agent - First aid kit - Warning notices - Contact notice for emergency services, National Poisons Centre etc. |  |  |  |  |

If there is one or more High Risk (H) actions needed, then the risk of injury could be high and immediate action should be taken.

Medium Risk (M) actions should be dealt with as soon as possible.   Low Risk (L) actions should be dealt with as soon as practicable.

Risk Assessment carried out by: 		Date:	/	/

© All Rights Reserved